CASO DE ESTUDIO

Kenneth Uohnson Rodriguez

CCNA PRIMER MODULO

PROFESOR: KEN CONTRERAS

FECHA DE ENTREGA: 23/06/2026

# INDICE

Contenido

INDICE	2

DIAGRAMA DE TOPOLOGÍA	3

TABLAS	4

VERIFICACION DE COMANDOS:	12

¿Que hace cada comando?	12

ROUTER 1:	13

ROUTER 2:	20

ROUTER 3:	26

ROUTER 4:	32

ROUTER 5:	38

ROUTER 6:	44

ROUTER 7:	50

# DIAGRAMA DE TOPOLOGÍA

# TABLAS

| Hosts Solicitados | Dirección de Red | Máscara de Subred | Máscara Wildcard | Primera IP Útil | Última IP Útil | Dirección de Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| 3.000 | 192.168.0.0 | 255.255.240.0 | 0.0.15.255 | 192.168.0.1 | 192.168.15.254 | 192.168.15.255 |
| 2.047 | 192.168.16.0 | 255.255.240.0 | 0.0.15.255 | 192.168.16.1 | 192.168.31.254 | 192.168.31.255 |
| 530 | 192.168.32.0 | 255.255.252.0 | 0.0.3.255 | 192.168.32.1 | 192.168.35.254 | 192.168.35.255 |
| 200 | 192.168.36.0 | 255.255.255.0 | 0.0.0.255 | 192.168.36.1 | 192.168.36.254 | 192.168.36.255 |
| 127 | 192.168.37.0 | 255.255.255.0 | 0.0.0.255 | 192.168.37.1 | 192.168.37.254 | 192.168.37.255 |
| 64 | 192.168.38.0 | 255.255.255.128 | 0.0.0.127 | 192.168.38.1 | 192.168.38.126 | 192.168.38.127 |

| Dispositivo | Interface | Red | Dirección IP | Máscara Subred | Gateway Por defecto |
| --- | --- | --- | --- | --- | --- |
| Router 1 | Gig0/0/0 | 10.0.0.20/30 | 10.0.0.21 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.36.0/24 | 192.168.36.1 | 255.255.255.0 | - |
|  | Se0/1/0 | 10.0.0.0/30 | 10.0.0.1 | 255.255.255.252 | - |
| Router 2 | Gig0/0/0 | 10.0.0.12/30 | 10.0.0.13 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.0.0/20 | 192.168.0.1 | 255.255.240.0 | - |
|  | Se0/1/0 | 10.0.0.4/30 | 10.0.0.5 | 255.255.255.252 | - |
|  | Se0/1/1 | 10.0.0.0/30 | 10.0.0.2 | 255.255.255.252 | - |
| Router 3 | Gig0/0/0 | 10.0.0.8/30 | 10.0.0.9 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.38.0/25 | 192.168.38.1 | 255.255.255.128 | - |
|  | Se0/1/0 | 10.0.0.4/30 | 10.0.0.6 | 255.255.255.252 | - |
|  | Se0/1/1 | 10.0.0.16/30 | 10.0.0.18 | 255.255.255.252 | - |
| Router 4 | Gig0/0/0 | 10.0.0.8/30 | 10.0.0.10 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.32.0/22 | 192.168.32.1 | 255.255.252.0 | - |
|  | Se0/1/0 | 10.0.0.32/30 | 10.0.0.34 | 255.255.255.252 | - |

| Dispositivo | Interface | Red | Dirección IP | Máscara Subred | Gateway Por defecto |
| --- | --- | --- | --- | --- | --- |
| Router 5 | Gig0/0/0 | 10.0.0.28/30 | 10.0.0.30 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.16.0/20 | 192.168.16.1 | 255.255.240.0 | - |
|  | Se0/1/0 | 10.0.0.36/30 | 10.0.0.38 | 255.255.255.252 | - |
|  | Se0/1/1 | 10.0.0.32/30 | 10.0.0.33 | 255.255.255.252 |  |
| Router 6 | Gig0/0/0 | 10.0.0.20/30 | 10.0.0.22 | 255.255.255.252 | - |
|  | Gig0/0/1 | 192.168.37.0/24 | 192.168.37.1 | 255.255.255.0 | - |
|  | Se0/1/0 | 10.0.0.24/30 | 10.0.0.25 | 255.255.255.252 | - |
|  | Se0/1/1 | 10.0.0.36/30 | 10.0.0.37 | 255.255.255.252 | - |
| Router 7 | Gig0/0/0 | 10.0.0.12/30 | 10.0.0.14 | 255.255.255.252 | - |
|  | Gig0/0/1 | 10.0.0.28/30 | 10.0.0.29 | 255.255.255.252 | - |
|  | Se0/1/0 | 10.0.0.24/30 | 10.0.0.26 | 255.255.255.252 | - |
|  | Se0/1/1 | 10.0.0.16/30 | 10.0.0.17 | 255.255.255.252 | - |

| Dispositivo | Interface | Red | Dirección IP | Máscara Subred | Gateway Por defecto |
| --- | --- | --- | --- | --- | --- |
| HTTP PROY | Fa0/2 | 192.168.36.0/24 | 192.168.36.10 | 255.255.255.0 | 192.168.36.1 |
| HTTP CCNA | Fa0/1 | 192.168.36.0/24 | 192.168.36.20 | 255.255.255.0 | 192.168.36.1 |
| PC0 | Fa0/2 | 192.168.0.0/20 | 192.168.0.2 | 255.255.240.0 | 192.168.0.1 |
| PC1 | Fa0/2 | 192.168.38.0/25 | 192.168.38.2 | 255.255.255.128 | 192.168.38.1 |
| TFTP | Fa0/3 | 192.168.32.0/22 | 192.168.32.2 | 255.255.252.0 | 192.168.32.1 |
| DNS | Fa0/2 | 192.168.32.0/22 | 192.168.32.5 | 255.255.252.0 | 192.168.32.1 |
| PC03 | Fa0/2 | 192.168.16.0/20 | 192.168.16.2 | 255.255.240.0 | 192.168.16.1 |
| IMPRESORA | Fa0/3 | 192.168.16.0/20 | 192.168.16.3 | 255.255.240.0 | 192.168.16.1 |
| LAPTOP0 | Fa0/3 | 192.168.37.0/24 | 192.168.37.3 | 255.255.255.0 | 192.168.37.1 |
| PC4 | Fa0/2 | 192.168.37.0/24 | 192.168.37.2 | 255.255.255.0 | 192.168.37.1 |

| Router | Red de Destino | Máscara de Subred | Siguiente Salto | Comando CLI Completo |
| --- | --- | --- | --- | --- |
| R1 | 10.0.0.0 | 255.255.255.252 | 10.0.0.2 | ip route 10.0.0.0 255.255.255.252 10.0.0.2 |
| R1 | 10.0.0.20 | 255.255.255.252 | 10.0.0.22 | ip route 10.0.0.20 255.255.255.252 10.0.0.22 |
| R1 | 192.168.0.0 | 255.255.240.0 | 10.0.0.2 | ip route 192.168.0.0 255.255.240.0 10.0.0.2 |
| R1 | 192.168.16.0 | 255.255.240.0 | 10.0.0.22 | ip route 192.168.16.0 255.255.240.0 10.0.0.22 |
| R1 | 192.168.32.0 | 255.255.252.0 | 10.0.0.2 | ip route 192.168.32.0 255.255.252.0 10.0.0.2 |
| R1 | 192.168.32.0 | 255.255.252.0 | 10.0.0.22 | ip route 192.168.32.0 255.255.252.0 10.0.0.22 |
| R1 | 192.168.37.0 | 255.255.255.0 | 10.0.0.22 | ip route 192.168.37.0 255.255.255.0 10.0.0.22 |
| R1 | 192.168.38.0 | 255.255.255.128 | 10.0.0.2 | ip route 192.168.38.0 255.255.255.128 10.0.0.2 |
| R2 | 10.0.0.0 | 255.255.255.252 | 10.0.0.1 | ip route 10.0.0.0 255.255.255.252 10.0.0.1 |
| R2 | 10.0.0.12 | 255.255.255.252 | 10.0.0.14 | ip route 10.0.0.12 255.255.255.252 10.0.0.14 |
| R2 | 10.0.0.16 | 255.255.255.252 | 10.0.0.18 | ip route 10.0.0.16 255.255.255.252 10.0.0.18 |
| R2 | 192.168.16.0 | 255.255.240.0 | 10.0.0.6 | ip route 192.168.16.0 255.255.240.0 10.0.0.6 |
| R2 | 192.168.32.0 | 255.255.252.0 | 10.0.0.6 | ip route 192.168.32.0 255.255.252.0 10.0.0.6 |
| R2 | 192.168.36.0 | 255.255.255.0 | 10.0.0.1 | ip route 192.168.36.0 255.255.255.0 10.0.0.1 |
| R2 | 192.168.37.0 | 255.255.255.0 | 10.0.0.1 | ip route 192.168.37.0 255.255.255.0 10.0.0.1 |
| R2 | 192.168.38.0 | 255.255.255.128 | 10.0.0.14 | ip route 192.168.38.0 255.255.255.128 10.0.0.14 |
| R2 | 192.168.38.0 | 255.255.255.128 | 10.0.0.6 | ip route 192.168.38.0 255.255.255.128 10.0.0.6 |
| R3 | 10.0.0.0 | 255.255.255.252 | 10.0.0.5 | ip route 10.0.0.0 255.255.255.252 10.0.0.5 |
| R3 | 10.0.0.4 | 255.255.255.252 | 10.0.0.5 | ip route 10.0.0.4 255.255.255.252 10.0.0.5 |
| R3 | 10.0.0.8 | 255.255.255.252 | 10.0.0.10 | ip route 10.0.0.8 255.255.255.252 10.0.0.10 |
| R3 | 10.0.0.16 | 255.255.255.252 | 10.0.0.17 | ip route 10.0.0.16 255.255.255.252 10.0.0.17 |
| R3 | 192.168.0.0 | 255.255.240.0 | 10.0.0.5 | ip route 192.168.0.0 255.255.240.0 10.0.0.5 |
| R3 | 192.168.16.0 | 255.255.240.0 | 10.0.0.17 | ip route 192.168.16.0 255.255.240.0 10.0.0.17 |
| R3 | 192.168.32.0 | 255.255.252.0 | 10.0.0.10 | ip route 192.168.32.0 255.255.252.0 10.0.0.10 |
| R3 | 192.168.36.0 | 255.255.255.0 | 10.0.0.5 | ip route 192.168.36.0 255.255.255.0 10.0.0.5 |
| R3 | 192.168.37.0 | 255.255.255.0 | 10.0.0.17 | ip route 192.168.37.0 255.255.255.0 10.0.0.17 |
| R4 | 10.0.0.0 | 255.255.255.252 | 10.0.0.9 | ip route 10.0.0.0 255.255.255.252 10.0.0.9 |
| R4 | 10.0.0.4 | 255.255.255.252 | 10.0.0.9 | ip route 10.0.0.4 255.255.255.252 10.0.0.9 |
| R4 | 10.0.0.20 | 255.255.255.252 | 10.0.0.33 | ip route 10.0.0.20 255.255.255.252 10.0.0.33 |
| R4 | 10.0.0.36 | 255.255.255.252 | 10.0.0.33 | ip route 10.0.0.36 255.255.255.252 10.0.0.33 |
| R4 | 192.168.0.0 | 255.255.240.0 | 10.0.0.9 | ip route 192.168.0.0 255.255.240.0 10.0.0.9 |
| R4 | 192.168.16.0 | 255.255.240.0 | 10.0.0.33 | ip route 192.168.16.0 255.255.240.0 10.0.0.33 |
| R4 | 192.168.36.0 | 255.255.255.0 | 10.0.0.9 | ip route 192.168.36.0 255.255.255.0 10.0.0.9 |
| R4 | 192.168.36.0 | 255.255.255.0 | 10.0.0.33 | ip route 192.168.36.0 255.255.255.0 10.0.0.33 |
| R4 | 192.168.37.0 | 255.255.255.0 | 10.0.0.33 | ip route 192.168.37.0 255.255.255.0 10.0.0.33 |
| R4 | 192.168.38.0 | 255.255.255.128 | 10.0.0.9 | ip route 192.168.38.0 255.255.255.128 10.0.0.9 |
| R5 | 10.0.0.0 | 255.255.255.252 | 10.0.0.37 | ip route 10.0.0.0 255.255.255.252 10.0.0.37 |
| R5 | 10.0.0.20 | 255.255.255.252 | 10.0.0.37 | ip route 10.0.0.20 255.255.255.252 10.0.0.37 |
| R5 | 10.0.0.24 | 255.255.255.252 | 10.0.0.27 | ip route 10.0.0.24 255.255.255.252 10.0.0.27 |
| R5 | 10.0.0.28 | 255.255.255.252 | 10.0.0.29 | ip route 10.0.0.28 255.255.255.252 10.0.0.29 |
| R5 | 10.0.0.32 | 255.255.255.252 | 10.0.0.34 | ip route 10.0.0.32 255.255.255.252 10.0.0.34 |
| R5 | 192.168.32.0 | 255.255.252.0 | 10.0.0.34 | ip route 192.168.32.0 255.255.252.0 10.0.0.34 |
| R5 | 192.168.36.0 | 255.255.255.0 | 10.0.0.37 | ip route 192.168.36.0 255.255.255.0 10.0.0.37 |
| R5 | 192.168.37.0 | 255.255.255.0 | 10.0.0.37 | ip route 192.168.37.0 255.255.255.0 10.0.0.37 |
| R5 | 192.168.38.0 | 255.255.255.128 | 10.0.0.34 | ip route 192.168.38.0 255.255.255.128 10.0.0.34 |
| R6 | 10.0.0.0 | 255.255.255.252 | 10.0.0.1 | ip route 10.0.0.0 255.255.255.252 10.0.0.1 |
| R6 | 10.0.0.24 | 255.255.255.252 | 10.0.0.26 | ip route 10.0.0.24 255.255.255.252 10.0.0.26 |
| R6 | 10.0.0.40 | 255.255.255.252 | 10.0.0.38 | ip route 10.0.0.40 255.255.255.252 10.0.0.38 |
| R6 | 192.168.0.0 | 255.255.240.0 | 10.0.0.29 | ip route 192.168.0.0 255.255.240.0 10.0.0.29 |
| R6 | 192.168.0.0 | 255.255.240.0 | 10.0.0.21 | ip route 192.168.0.0 255.255.240.0 10.0.0.21 |
| R6 | 192.168.16.0 | 255.255.240.0 | 10.0.0.38 | ip route 192.168.16.0 255.255.240.0 10.0.0.38 |
| R6 | 192.168.32.0 | 255.255.252.0 | 10.0.0.38 | ip route 192.168.32.0 255.255.252.0 10.0.0.38 |
| R6 | 192.168.36.0 | 255.255.255.0 | 10.0.0.21 | ip route 192.168.36.0 255.255.255.0 10.0.0.21 |
| R6 | 192.168.38.0 | 255.255.255.128 | 10.0.0.21 | ip route 192.168.38.0 255.255.255.128 10.0.0.21 |
| R7 | 10.0.0.0 | 255.255.255.252 | 10.0.0.13 | ip route 10.0.0.0 255.255.255.252 10.0.0.13 |
| R7 | 10.0.0.12 | 255.255.255.252 | 10.0.0.13 | ip route 10.0.0.12 255.255.255.252 10.0.0.13 |
| R7 | 10.0.0.16 | 255.255.255.252 | 10.0.0.9 | ip route 10.0.0.16 255.255.255.252 10.0.0.9 |
| R7 | 10.0.0.16 | 255.255.255.252 | 10.0.0.18 | ip route 10.0.0.16 255.255.255.252 10.0.0.18 |
| R7 | 10.0.0.24 | 255.255.255.252 | 10.0.0.25 | ip route 10.0.0.24 255.255.255.252 10.0.0.25 |
| R7 | 10.0.0.28 | 255.255.255.252 | 10.0.0.30 | ip route 10.0.0.28 255.255.255.252 10.0.0.30 |
| R7 | 192.168.0.0 | 255.255.240.0 | 10.0.0.13 | ip route 192.168.0.0 255.255.240.0 10.0.0.13 |
| R7 | 192.168.16.0 | 255.255.240.0 | 10.0.0.25 | ip route 192.168.16.0 255.255.240.0 10.0.0.25 |
| R7 | 192.168.32.0 | 255.255.252.0 | 10.0.0.18 | ip route 192.168.32.0 255.255.252.0 10.0.0.18 |
| R7 | 192.168.37.0 | 255.255.255.0 | 10.0.0.25 | ip route 192.168.37.0 255.255.255.0 10.0.0.25 |
| R7 | 192.168.38.0 | 255.255.255.128 | 10.0.0.18 | ip route 192.168.38.0 255.255.255.128 10.0.0.18 |
| R7 | 192.168.38.0 | 255.255.255.128 | 10.0.0.30 | ip route 192.168.38.0 255.255.255.128 10.0.0.30 |

# VERIFICACION DE COMANDOS:

## ¿Que hace cada comando?

- SHOW IP INTERFACE BRIEF

Resume rápidamente el estado y configuración de todas las interfaces del dispositivo.

Es el primer comando para diagnósticos básicos.

- SHOW RUNNING-CONFIG

Muestra la configuración que se está ejecutando justo en el mismo momento en la memoria RAM.

- SHOW IP ROUTE

Muestra la tabla de enrutamiento IP del router, permite ver la lista de rutas que se configuran.

- SHOW CDP NEIGHBOR

Muestra los dispositivos Cisco que estén conectados directamente al equipo utilizando el protocolo CDP.

- SHOW IP DHCP BINDING

Muestra la lista de asignaciones de direcciones IP que el router entrega a los hosts por medio del protocolo DHCP.

# ROUTER 1:

R1#SHOW IP INTERFACE BRIEF

R1#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1588 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R1

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

ip dhcp excluded-address 192.168.36.10

ip dhcp excluded-address 192.168.36.20

!

ip dhcp pool LAN

network 192.168.36.0 255.255.255.0

default-router 192.168.36.1

dns-server 192.168.32.5

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R6 PARA IP ROUTE

ip address 10.0.0.21 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.36.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R2 PARA IP ROUTE

ip address 10.0.0.1 255.255.255.252

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.38.0 255.255.255.128 10.0.0.2

ip route 192.168.0.0 255.255.240.0 10.0.0.2

ip route 192.168.32.0 255.255.252.0 10.0.0.2

ip route 192.168.37.0 255.255.255.0 10.0.0.22

ip route 192.168.16.0 255.255.240.0 10.0.0.22

ip route 192.168.32.0 255.255.252.0 10.0.0.22

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R#1SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C 10.0.0.0/30 is directly connected, Serial0/1/0

L 10.0.0.1/32 is directly connected, Serial0/1/0

C 10.0.0.20/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.21/32 is directly connected, GigabitEthernet0/0/0

S 192.168.0.0/20 [1/0] via 10.0.0.2

S 192.168.16.0/20 [1/0] via 10.0.0.22

S 192.168.32.0/22 [1/0] via 10.0.0.22

[1/0] via 10.0.0.2

192.168.36.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.36.0/24 is directly connected, GigabitEthernet0/0/1

L 192.168.36.1/32 is directly connected, GigabitEthernet0/0/1

S 192.168.37.0/24 [1/0] via 10.0.0.22

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.2

R1#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 4 subnets, 2 masks

C 10.0.0.0/30 is directly connected, Serial0/1/0

L 10.0.0.1/32 is directly connected, Serial0/1/0

C 10.0.0.20/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.21/32 is directly connected, GigabitEthernet0/0/0

S 192.168.0.0/20 [1/0] via 10.0.0.2

S 192.168.16.0/20 [1/0] via 10.0.0.22

S 192.168.32.0/22 [1/0] via 10.0.0.22

[1/0] via 10.0.0.2

192.168.36.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.36.0/24 is directly connected, GigabitEthernet0/0/1

L 192.168.36.1/32 is directly connected, GigabitEthernet0/0/1

S 192.168.37.0/24 [1/0] via 10.0.0.22

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.2

R1#

R1#

R1#

R1#Show cdp neighbor

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 139 S 2960 Gig 0/1

R2 Ser 0/1/0 139 R ISR4300 Ser 0/1/1

R6 Gig 0/0/0 139 R ISR4300 Gig 0/0/0

# ROUTER 2:

R2#SHOW IP INTERFACE BRIEF

R2#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1707 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R2

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

!

ip dhcp pool LAN

network 192.168.0.0 255.255.240.0

default-router 192.168.0.1

dns-server 192.168.32.5

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R7 PARA IP ROUTE

ip address 10.0.0.13 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.0.1 255.255.240.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ HACIA R3

ip address 10.0.0.5 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

description INTERFAZ AL R1 PARA IP ROUTE

ip address 10.0.0.2 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.16.0 255.255.240.0 10.0.0.6

ip route 192.168.38.0 255.255.255.128 10.0.0.14

ip route 192.168.38.0 255.255.255.128 10.0.0.6

ip route 192.168.32.0 255.255.252.0 10.0.0.6

ip route 192.168.36.0 255.255.255.0 10.0.0.1

ip route 192.168.37.0 255.255.255.0 10.0.0.1

ip route 10.0.0.12 255.255.255.252 10.0.0.14

ip route 10.0.0.0 255.255.255.252 10.0.0.1

ip route 10.0.0.16 255.255.255.252 10.0.0.18

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

End

R2#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 6 subnets, 2 masks

C 10.0.0.0/30 is directly connected, Serial0/1/1

L 10.0.0.2/32 is directly connected, Serial0/1/1

C 10.0.0.4/30 is directly connected, Serial0/1/0

L 10.0.0.5/32 is directly connected, Serial0/1/0

C 10.0.0.12/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.13/32 is directly connected, GigabitEthernet0/0/0

C 192.168.0.0/20 is directly connected, GigabitEthernet0/0/1

192.168.0.0/32 is subnetted, 1 subnets

L 192.168.0.1/32 is directly connected, GigabitEthernet0/0/1

S 192.168.16.0/20 [1/0] via 10.0.0.6

S 192.168.32.0/22 [1/0] via 10.0.0.6

S 192.168.36.0/24 [1/0] via 10.0.0.1

S 192.168.37.0/24 [1/0] via 10.0.0.1

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.14

[1/0] via 10.0.0.6

R2#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 152 S 2960 Fas 0/1

R1 Ser 0/1/1 160 R ISR4300 Ser 0/1/0

R3 Ser 0/1/0 161 R ISR4300 Ser 0/1/0

R7 Gig 0/0/0 161 R ISR4300 Gig 0/0/0

R2#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 152 S 2960 Fas 0/1

R1 Ser 0/1/1 160 R ISR4300 Ser 0/1/0

R3 Ser 0/1/0 161 R ISR4300 Ser 0/1/0

R7 Gig 0/0/0 161 R ISR4300 Gig 0/0/0

R2#

R2#Show ip dhcp binding

IP address Client-ID/ Lease expiration Type

Hardware address

192.168.0.2 00D0.FF15.D7B1 – Automatic

# ROUTER 3:

R3#SHOW IP INTERFACE BRIEF

R3#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1699 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R3

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

!

ip dhcp pool LAN

network 192.168.38.0 255.255.255.128

default-router 192.168.38.1

dns-server 192.168.32.5

!

!

!

no ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R4 PARA IP ROUTE

ip address 10.0.0.9 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.38.1 255.255.255.128

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R2 PARA IP ROUTE

ip address 10.0.0.6 255.255.255.252

!

interface Serial0/1/1

description INTERFAZ AL R3 PARA IP ROUTE

ip address 10.0.0.18 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.16.0 255.255.240.0 10.0.0.17

ip route 192.168.37.0 255.255.255.0 10.0.0.17

ip route 192.168.0.0 255.255.240.0 10.0.0.5

ip route 192.168.32.0 255.255.252.0 10.0.0.10

ip route 192.168.36.0 255.255.255.0 10.0.0.5

ip route 10.0.0.8 255.255.255.252 10.0.0.10

ip route 10.0.0.4 255.255.255.252 10.0.0.5

ip route 10.0.0.16 255.255.255.252 10.0.0.17

ip route 10.0.0.0 255.255.255.252 10.0.0.5

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R3#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

S 10.0.0.0/30 [1/0] via 10.0.0.5

C 10.0.0.4/30 is directly connected, Serial0/1/0

L 10.0.0.6/32 is directly connected, Serial0/1/0

C 10.0.0.8/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.9/32 is directly connected, GigabitEthernet0/0/0

C 10.0.0.16/30 is directly connected, Serial0/1/1

L 10.0.0.18/32 is directly connected, Serial0/1/1

S 192.168.0.0/20 [1/0] via 10.0.0.5

S 192.168.16.0/20 [1/0] via 10.0.0.17

S 192.168.32.0/22 [1/0] via 10.0.0.10

S 192.168.36.0/24 [1/0] via 10.0.0.5

S 192.168.37.0/24 [1/0] via 10.0.0.17

192.168.38.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.38.0/25 is directly connected, GigabitEthernet0/0/1

L 192.168.38.1/32 is directly connected, GigabitEthernet0/0/1

R3#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 167 S 2960 Fas 0/1

R2 Ser 0/1/0 167 R ISR4300 Ser 0/1/0

R4 Gig 0/0/0 176 R ISR4300 Gig 0/0/0

R7 Ser 0/1/1 176 R ISR4300 Ser 0/1/1

R3#SHOW IP DHCP BINDING

IP address Client-ID/ Lease expiration Type

Hardware address

192.168.38.2 0002.4A73.31C2 – Automatic

# ROUTER 4:

R4#SHOW IP INTERFACE BRIEF

R4#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1809 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R4

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

ip dhcp excluded-address 192.168.32.2

ip dhcp excluded-address 192.168.32.5

!

ip dhcp pool LAN

network 192.168.32.0 255.255.252.0

default-router 192.168.32.1

dns-server 192.168.32.5

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R4 PARA IP ROUTE

ip address 10.0.0.10 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.32.1 255.255.252.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R4 PARA IP ROUTE

ip address 10.0.0.34 255.255.255.252

!

interface Serial0/1/1

no ip address

clock rate 2000000

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.38.0 255.255.255.128 10.0.0.9

ip route 192.168.36.0 255.255.255.0 10.0.0.9

ip route 192.168.0.0 255.255.240.0 10.0.0.9

ip route 192.168.37.0 255.255.255.0 10.0.0.33

ip route 192.168.16.0 255.255.240.0 10.0.0.33

ip route 192.168.36.0 255.255.255.0 10.0.0.33

ip route 10.0.0.0 255.255.255.252 10.0.0.9

ip route 10.0.0.20 255.255.255.252 10.0.0.33

ip route 10.0.0.4 255.255.255.252 10.0.0.9

ip route 10.0.0.36 255.255.255.252 10.0.0.33

ip route 10.0.0.16 255.255.255.252 10.0.0.9

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

End

R4#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 2 masks

S 10.0.0.0/30 [1/0] via 10.0.0.9

S 10.0.0.4/30 [1/0] via 10.0.0.9

C 10.0.0.8/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.10/32 is directly connected, GigabitEthernet0/0/0

S 10.0.0.16/30 [1/0] via 10.0.0.9

S 10.0.0.20/30 [1/0] via 10.0.0.33

C 10.0.0.32/30 is directly connected, Serial0/1/0

L 10.0.0.34/32 is directly connected, Serial0/1/0

S 10.0.0.36/30 [1/0] via 10.0.0.33

S 192.168.0.0/20 [1/0] via 10.0.0.9

S 192.168.16.0/20 [1/0] via 10.0.0.33

C 192.168.32.0/22 is directly connected, GigabitEthernet0/0/1

192.168.32.0/32 is subnetted, 1 subnets

L 192.168.32.1/32 is directly connected, GigabitEthernet0/0/1

S 192.168.36.0/24 [1/0] via 10.0.0.9

[1/0] via 10.0.0.33

S 192.168.37.0/24 [1/0] via 10.0.0.33

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.9

R4#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 162 S 2960 Fas 0/1

R3 Gig 0/0/0 171 R ISR4300 Gig 0/0/0

R5 Ser 0/1/0 168 R ISR4300 Ser 0/1/1

# ROUTER 5:

R5#SHOW IP INTERFACE BRIEF

R5#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1786 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R5

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

ip dhcp excluded-address 192.168.16.3

!

ip dhcp pool LAN

network 192.168.16.0 255.255.240.0

default-router 192.168.16.1

dns-server 192.168.32.5

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R7 PARA IP ROUTE

ip address 10.0.0.30 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.16.1 255.255.240.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R6 PARA IP ROUTE

ip address 10.0.0.38 255.255.255.252

!

interface Serial0/1/1

description INTERFAZ AL R4 PARA IP ROUTE

ip address 10.0.0.33 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.0.0 255.255.240.0 10.0.0.29

ip route 192.168.38.0 255.255.255.128 10.0.0.34

ip route 192.168.32.0 255.255.252.0 10.0.0.34

ip route 192.168.37.0 255.255.255.0 10.0.0.37

ip route 192.168.36.0 255.255.255.0 10.0.0.37

ip route 10.0.0.32 255.255.255.252 10.0.0.34

ip route 10.0.0.28 255.255.255.252 10.0.0.29

ip route 10.0.0.24 255.255.255.252 10.0.0.27

ip route 10.0.0.0 255.255.255.252 10.0.0.37

ip route 10.0.0.20 255.255.255.252 10.0.0.37

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R5#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

S 10.0.0.0/30 [1/0] via 10.0.0.37

S 10.0.0.20/30 [1/0] via 10.0.0.37

C 10.0.0.28/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.30/32 is directly connected, GigabitEthernet0/0/0

C 10.0.0.32/30 is directly connected, Serial0/1/1

L 10.0.0.33/32 is directly connected, Serial0/1/1

C 10.0.0.36/30 is directly connected, Serial0/1/0

L 10.0.0.38/32 is directly connected, Serial0/1/0

S 192.168.0.0/20 [1/0] via 10.0.0.29

C 192.168.16.0/20 is directly connected, GigabitEthernet0/0/1

192.168.16.0/32 is subnetted, 1 subnets

L 192.168.16.1/32 is directly connected, GigabitEthernet0/0/1

S 192.168.32.0/22 [1/0] via 10.0.0.34

S 192.168.36.0/24 [1/0] via 10.0.0.37

S 192.168.37.0/24 [1/0] via 10.0.0.37

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.34

R5#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 178 S 2960 Fas 0/1

R4 Ser 0/1/1 127 R ISR4300 Ser 0/1/0

R6 Ser 0/1/0 178 R ISR4300 Ser 0/1/1

R7 Gig 0/0/0 127 R ISR4300 Gig 0/0/1

R5#SHOW IP DHCP BINDING

IP address Client-ID/ Lease expiration Type

Hardware address

192.168.16.2 0060.5C66.452D – Automatic

# ROUTER 6:

R6#SHOW IP INTERFACE BRIEF

R6#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1675 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R6

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

!

ip dhcp pool LAN

network 192.168.37.0 255.255.255.0

default-router 192.168.37.1

dns-server 192.168.32.5

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R1 PARA IP ROUTE

ip address 10.0.0.22 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ DEL GATEWAY .1

ip address 192.168.37.1 255.255.255.0

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R7 PARA IP ROUTE

ip address 10.0.0.25 255.255.255.252

clock rate 2000000

!

interface Serial0/1/1

description INTERFAZ AL R5 PARA IP ROUTE

ip address 10.0.0.37 255.255.255.252

clock rate 2000000

!

interface Vlan1

no ip address

shutdown

!

ip classless

ip route 192.168.16.0 255.255.240.0 10.0.0.38

ip route 192.168.38.0 255.255.255.128 10.0.0.21

ip route 192.168.32.0 255.255.252.0 10.0.0.38

ip route 192.168.36.0 255.255.255.0 10.0.0.21

ip route 192.168.0.0 255.255.240.0 10.0.0.21

ip route 10.0.0.40 255.255.255.252 10.0.0.38

ip route 10.0.0.0 255.255.255.252 10.0.0.1

ip route 10.0.0.24 255.255.255.252 10.0.0.26

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R6#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

C 10.0.0.20/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.22/32 is directly connected, GigabitEthernet0/0/0

C 10.0.0.24/30 is directly connected, Serial0/1/0

L 10.0.0.25/32 is directly connected, Serial0/1/0

C 10.0.0.36/30 is directly connected, Serial0/1/1

L 10.0.0.37/32 is directly connected, Serial0/1/1

S 10.0.0.40/30 [1/0] via 10.0.0.38

S 192.168.0.0/20 [1/0] via 10.0.0.21

S 192.168.16.0/20 [1/0] via 10.0.0.38

S 192.168.32.0/22 [1/0] via 10.0.0.38

S 192.168.36.0/24 [1/0] via 10.0.0.21

192.168.37.0/24 is variably subnetted, 2 subnets, 2 masks

C 192.168.37.0/24 is directly connected, GigabitEthernet0/0/1

L 192.168.37.1/32 is directly connected, GigabitEthernet0/0/1

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.21

R6#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

Switch Gig 0/0/1 121 S 2960 Fas 0/1

R1 Gig 0/0/0 129 R ISR4300 Gig 0/0/0

R5 Ser 0/1/1 128 R ISR4300 Ser 0/1/0

R7 Ser 0/1/0 130 R ISR4300 Ser 0/1/0

R6#SHOW IP DHCP BINDING

IP address Client-ID/ Lease expiration Type

Hardware address

192.168.37.2 0005.5EC7.C6CB -- Automatic

192.168.37.3 0001.C99B.7DE1 – Automatic

# ROUTER 7:

R7#SHOW IP INTERFACE BRIEF

R7#SHOW RUNNING-CONFIG

Building configuration...

Current configuration : 1458 bytes

!

version 15.4

no service timestamps log datetime msec

no service timestamps debug datetime msec

service password-encryption

!

hostname R7

!

!

!

enable secret 5 $1$mERr$CunnQjkAEasAf02ni13tK/

!

!

!

!

!

!

ip cef

no ipv6 cef

!

!

!

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0/0

description INTERFAZ AL R2 PARA IP ROUTE

ip address 10.0.0.14 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/1

description INTERFAZ AL R6 PARA IP ROUTE

ip address 10.0.0.29 255.255.255.252

duplex auto

speed auto

!

interface GigabitEthernet0/0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Serial0/1/0

description INTERFAZ AL R6 PARA IP ROUTE

ip address 10.0.0.26 255.255.255.252

!

interface Serial0/1/1

description INTERFAZ AL R3 PARA IP ROUTE

ip address 10.0.0.17 255.255.255.252

!

interface Vlan1

no ip address

shutdown

!

router rip

!

ip classless

ip route 192.168.16.0 255.255.240.0 10.0.0.25

ip route 192.168.0.0 255.255.240.0 10.0.0.13

ip route 192.168.38.0 255.255.255.128 10.0.0.18

ip route 192.168.37.0 255.255.255.0 10.0.0.25

ip route 192.168.38.0 255.255.255.128 10.0.0.30

ip route 192.168.32.0 255.255.252.0 10.0.0.18

!

ip flow-export version 9

!

!

!

banner motd ^C ACCESO RESTRINGIDO: Solo personal autorizado. Las conexiones son monitoreadas. ^C

!

!

!

!

line con 0

password 7 0822455D0A16

login

!

line aux 0

!

line vty 0 4

password 7 0822455D0A16

login

!

!

!

End

R7#SHOW IP ROUTE

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

C 10.0.0.12/30 is directly connected, GigabitEthernet0/0/0

L 10.0.0.14/32 is directly connected, GigabitEthernet0/0/0

C 10.0.0.16/30 is directly connected, Serial0/1/1

L 10.0.0.17/32 is directly connected, Serial0/1/1

C 10.0.0.24/30 is directly connected, Serial0/1/0

L 10.0.0.26/32 is directly connected, Serial0/1/0

C 10.0.0.28/30 is directly connected, GigabitEthernet0/0/1

L 10.0.0.29/32 is directly connected, GigabitEthernet0/0/1

S 192.168.0.0/20 [1/0] via 10.0.0.13

S 192.168.16.0/20 [1/0] via 10.0.0.25

S 192.168.32.0/22 [1/0] via 10.0.0.18

S 192.168.37.0/24 [1/0] via 10.0.0.25

192.168.38.0/25 is subnetted, 1 subnets

S 192.168.38.0/25 [1/0] via 10.0.0.30

[1/0] via 10.0.0.18

R7#SHOW CDP NEIGHBOR

Capability Codes: R - Router, T - Trans Bridge, B - Source Route Bridge

S - Switch, H - Host, I - IGMP, r - Repeater, P - Phone

Device ID Local Intrfce Holdtme Capability Platform Port ID

R2 Gig 0/0/0 171 R ISR4300 Gig 0/0/0

R3 Ser 0/1/1 120 R ISR4300 Ser 0/1/1

R6 Ser 0/1/0 171 R ISR4300 Ser 0/1/0

R5 Gig 0/0/1 177 R ISR4300 Gig 0/0/0